AM Radio Kit – Assessment Rubric (100 Points)

(Based on Superheterodyne AM Receiver Construction & Testing)

1. Engineering Build Quality (25 Points)

(Aligned with soldering standards and assembly requirements in manual pages 1, 5–6)

1.1 Soldering Quality (15 pts)

| Level | Descriptor |
| --- | --- |
| Exemplary (15) | All joints meet IPC-style standards: smooth, properly wetted, concave fillets, no voids. No bridges, cold joints, or thermal damage. Demonstrates full understanding of heat transfer and solder flow principles. |
| Proficient (12) | Minor cosmetic imperfections; all joints electrically sound. No rework required. |
| Developing (9) | Multiple joints show poor wetting or excess solder; minor rework required. |
| Beginning (5) | Significant soldering defects (cold joints, bridges, insufficient heat). Circuit reliability compromised. |

1.2 Component Placement & Polarization (10 pts)

| Level | Descriptor |
| --- | --- |
| Exemplary (10) | All polarized components (electrolytics, diodes, transistors, IC) correctly oriented per schematic and PCB legend. Layout reflects professional assembly standards. |
| Proficient (8) | Minor placement/orientation issues corrected independently. |
| Developing (6) | Multiple placement errors requiring instructor intervention. |
| Beginning (3) | Incorrect installation of critical components; circuit non-functional or unsafe. |

2. Functional Performance – AM Receiver (25 Points)

(Aligned with superheterodyne operation and tuning stages described on page 5)

2.1 Signal Reception & Tuning (15 pts)

| Level | Descriptor |
| --- | --- |
| Exemplary (15) | Receives multiple AM stations clearly across band; tuning is selective with defined peaks; demonstrates proper IF alignment behavior (~455 kHz system response). |
| Proficient (12) | Receives ≥2 stations; tuning functional but broader peaks. |
| Developing (9) | Limited reception (1 station); weak selectivity or instability. |
| Beginning (5) | No usable reception; improper mixer/oscillator/IF function. |

2.2 Audio Output Quality (10 pts)

| Level | Descriptor |
| --- | --- |
| Exemplary (10) | Clean, undistorted audio; appropriate gain; no clipping at nominal levels; demonstrates proper coupling and amplification. |
| Proficient (8) | Minor distortion or noise at higher output levels. |
| Developing (6) | Noticeable distortion, weak amplification. |
| Beginning (3) | Audio unintelligible or absent. |

3. Static Electrical Verification (20 Points)

(Directly aligned with manual testing procedures – page 10–11)

3.1 Idle Current Measurement (10 pts)

| Level | Descriptor |
| --- | --- |
| Exemplary (10) | ≤ 10 mA (ideal range per manual); demonstrates understanding of quiescent current and power consumption. |
| Proficient (8) | 10–15 mA; slightly elevated but acceptable. |
| Developing (6) | 15–20 mA; indicates inefficiency or bias issue. |
| Beginning (3) | >20 mA; indicates fault (short, incorrect bias, or assembly error). |

3.2 Output Bias Voltage (TP1) (10 pts)

| Level | Descriptor |
| --- | --- |
| Exemplary (10) | 3.0–6.0 V stable (centered bias as specified). Demonstrates correct Class AB biasing. |
| Proficient (8) | Slight deviation; circuit functional. |
| Developing (6) | Outside optimal range; performance degraded. |
| Beginning (3) | Incorrect bias (<2V or >7V); amplifier improperly biased. |

4. Dynamic Performance & Measurement (20 Points)

(Based on gain, bandwidth, and distortion testing – pages 11–14)

4.1 AC Gain Measurement (10 pts)

| Level | Descriptor |
| --- | --- |
| Exemplary (10) | Gain measured and calculated within expected range (≈100–180). Demonstrates correct test setup and calculation (Vout/Vin). |
| Proficient (8) | Gain measured but slight deviation or minor calculation errors. |
| Developing (6) | Inconsistent measurement technique or incorrect interpretation. |
| Beginning (3) | Unable to measure or calculate gain correctly. |

4.2 Frequency Response / Bandwidth (10 pts)

| Level | Descriptor |
| --- | --- |
| Exemplary (10) | Correctly identifies 3 dB cutoff points; bandwidth >30 kHz as expected. Demonstrates proper oscilloscope use. |
| Proficient (8) | Reasonable measurement with minor inaccuracies. |
| Developing (6) | Partial understanding; incomplete or inaccurate measurements. |
| Beginning (3) | No meaningful bandwidth measurement. |

5. Engineering Analysis & Troubleshooting (10 Points)

5.1 Diagnostic Reasoning (10 pts)

| Level | Descriptor |
| --- | --- |
| Exemplary (10) | Demonstrates strong understanding of circuit stages (RF, IF, detector, audio). Can diagnose faults using measurements and theory. |
| Proficient (8) | Identifies basic faults and applies logical troubleshooting steps. |
| Developing (6) | Relies on trial-and-error; limited theoretical connection. |
| Beginning (3) | Unable to diagnose or explain circuit behavior. |

Final Score: /100